PART VI — CHAPTER 3: COMPONENT SELECTION

Now we reach one of the most important skills for a Robotics Engineer — Hardware Integrations:

Given a requirement, how do I choose the right component?

A beginner often thinks:

"Which component is the best?"

An engineer asks:

"Which component is appropriate for this requirement?"

There is no universally "best" motor, microcontroller, sensor, or communication protocol.

The right choice depends on the system.

1. The Component-Selection Chain

Let's establish the process first.

Suppose we're designing a robotic arm.

We need:

Move a 2 kg payload through a particular range of motion.

We don't start by searching:

"Best robotic motor."

Instead:

Requirement

↓

Physical analysis

↓

Speed / torque / force

↓

Operating conditions

↓

Component class

↓

Specific component

↓

Datasheet verification

↓

Integration

↓

Testing

This process applies to almost everything.

2. Choosing a Motor

Before selecting a motor, identify at least:

1. Required speed

How fast must it rotate?

2. Required torque

How much rotational force is required?

3. Voltage

What power supply is available?

4. Current

Can the power system provide the required current?

5. Duty cycle

Does it operate:

- continuously?

- intermittently?

- short bursts?

6. Feedback

Do we need:

- no feedback?

- speed feedback?

- position feedback?

7. Environment

Does it operate in:

- heat?

- dust?

- moisture?

- vibration?

3. Four Motor Categories You Should Know

For robotics, you should be comfortable with:

DC Brushed

BLDC

Stepper

Servo

But here's something important:

"Servo" isn't really one specific motor technology.

A servo system is fundamentally:

motor + feedback + controller

So a servo can use different motor technologies.

Let's build this carefully.

4. Brushed DC Motor

A basic brushed DC motor looks conceptually like:

Electrical power

↓

Motor

↓

Rotation

Change voltage/PWM:

change speed.

Reverse polarity:

change direction.

They're:

- simple

- inexpensive

- easy to control

- widely available

But they have:

brushes and a commutator

which physically wear.

5. Why Brushes Matter

Inside the motor, brushes transfer electrical current to the rotating part.

That creates:

- friction

- electrical arcing

- wear

- electromagnetic noise

So brushed motors can require maintenance in demanding applications.

But they're still extremely useful.

For example:

- small mobile robots

- educational robots

- pumps

- actuators

- simple mechanisms

Your familiar:

TT gear motor

is essentially a small brushed DC geared motor.

6. Why Add a Gearbox?

A small DC motor may spin extremely fast.

For example:

10,000 RPM

But your robot wheel might only need:

100 RPM.

A gearbox converts:

High speed

+

Low torque

↓

Low speed

+

Higher torque

Ignoring losses:

In reality:

because of gearbox losses.

7. Gear Ratio

Suppose:

Motor:

Gear ratio:

Output speed approximately:

That's exactly the sort of calculation we performed in the previous chapter.

8. But There's a Tradeoff

A larger reduction ratio gives:

lower speed

but:

greater output torque.

Suppose the motor produces:

and the gearbox is:

Ignoring losses:

With 80% efficiency:

So the gearbox dramatically changes what the motor can do mechanically.

9. BLDC Motors

Now let's move to:

Brushless DC motors

Instead of mechanical brushes, electronic switching controls the motor's phases.

Conceptually:

Battery

↓

Electronic Speed Controller

↓

BLDC Motor

↓

Rotation

BLDC motors are attractive because they can offer:

- high efficiency

- high power density

- long life

- high speed

- good thermal characteristics

They're common in:

- drones

- electric vehicles

- robotic joints

- industrial machines

10. But BLDC Adds Complexity

A brushed motor can be relatively simple:

PWM

↓

H-bridge

↓

Motor

A BLDC motor needs controlled commutation.

The controller needs to know the rotor's position, either through:

- Hall sensors

- encoder

- back-EMF sensing

- other rotor-position estimation methods

Then it energizes the appropriate phases.

So:

BLDC gives you performance, but the control system becomes more sophisticated.

11. Stepper Motors

Now consider:

Stepper motors

A stepper motor moves in discrete angular increments.

For example:

Step 1

↓

Step 2

↓

Step 3

↓

Step 4

Instead of simply saying:

"Rotate."

we can command:

"Take 200 steps."

A common stepper might have:

1.8° per full step.

Since:

we get:

200 full steps per revolution.

12. Why Steppers Are Useful

They are excellent when you need:

- predictable incremental movement

- relatively simple positioning

- high holding torque

- low-cost positioning systems

Examples:

- 3D printers

- CNC machines

- positioning mechanisms

13. The Stepper Trap

Here's something important.

A stepper motor can lose steps.

Suppose the controller commands:

1000 steps

but the motor encounters too much load.

It may physically move only:

900 steps

The controller may still believe:

"I moved 1000 steps."

That's an open-loop problem.

You can add an encoder to create a closed-loop stepper system.

This is another example of:

Feedback changing system behavior.

14. Servo Systems

Now we're ready for servos.

A servo system typically has:

Command

↓

Controller

↓

Motor

↓

Mechanical output

↓

Encoder

↓

Feedback

└────────────→ Controller

This is closed-loop control.

That's why servo systems are excellent for:

- precise positioning

- velocity control

- torque control

- robotic joints

15. Your MG996R Example

You've used:

MG996R servo

Think about what's actually inside it.

Conceptually:

PWM command

↓

Servo electronics

↓

DC motor

↓

Gearbox

↓

Output shaft

↓

Position feedback

└────────→ Controller

So when you send a servo command:

"Go to this angle."

you're not directly controlling the motor voltage.

You're commanding a closed-loop actuator system.

That's a very important distinction.

16. Motor Selection Decision

Let's simplify the selection process.

Brushed DC

Choose when you want:

- simplicity

- low cost

- moderate performance

BLDC

Choose when you need:

- high efficiency

- high power density

- high speed

- long life

Stepper

Choose when you need:

- discrete positioning

- predictable step motion

- high holding torque

- relatively simple positioning

Servo

Choose when you need:

- precise closed-loop control

- position/velocity/torque control

- dynamic performance

But remember:

The application determines the choice.

17. Next: Choosing the Gearbox

Suppose we've decided:

"I need a geared actuator."

What gearbox should we use?

Common types include:

- spur

- planetary

- worm

- harmonic/strain-wave

Let's understand why they exist.

18. Spur Gearbox

A spur gear transmission is relatively simple:

Small gear

↓

Large gear

Advantages:

- simple

- inexpensive

- efficient

- easy to manufacture

Disadvantages:

- backlash

- noise

- limited reduction per stage

Very common in:

- small robots

- toys

- simple actuators

19. Planetary Gearbox

A planetary gearbox has:

- sun gear

- planet gears

- ring gear

Conceptually:

Ring Gear

┌──────────────┐

│   ○  ○  ○    │

│      ●       │

└──────────────┘

Sun

Planetary gearboxes can provide:

- high torque density

- compact size

- good efficiency

- relatively high reduction

They're common in higher-performance robotic actuators.

20. Worm Gearbox

A worm drive uses:

Worm

↓

Worm wheel

A major advantage can be:

high reduction in a compact arrangement

and depending on geometry, some worm drives can resist back-driving.

This can be useful for:

- lifts

- positioning mechanisms

- certain actuators

But worm gears can have:

relatively high losses

compared with some other gearbox types.

21. Harmonic / Strain-Wave Gearboxes

These are particularly interesting for robotics.

They're commonly used where you need:

- very high reduction

- compact packaging

- high precision

- low backlash

They're popular in:

- robotic arms

- precision joints

- humanoid robots

They are more sophisticated and typically more expensive.

22. Gearbox Selection

Again, don't memorize:

"Planetary is better."

Instead ask:

Required torque?

Required speed?

Required backlash?

Required efficiency?

Required size?

Required lifetime?

Required precision?

Required cost?

Then choose.

23. Now: Motor Driver Selection

We have a motor.

The motor needs power.

The MCU needs control.

We need something between them.

That's the:

motor driver

For a brushed DC motor, we may use an:

H-bridge

For a BLDC motor:

ESC / BLDC controller

For a stepper:

stepper driver

For an industrial servo:

servo drive

24. H-Bridge

An H-bridge lets us control the polarity applied to a motor.

Conceptually:

+V

/  \

S1  S2

│    │

├Motor┤

│    │

S3  S4

\  /

GND

By switching different transistor combinations, we can achieve:

Forward

Reverse

Brake

Coast

The actual switching scheme depends on the driver.

25. What Should You Check on a Motor Driver Datasheet?

At minimum:

Voltage

Can it handle the motor voltage?

Current

Can it handle:

- continuous current?

- peak current?

- stall current?

Logic

Does it accept:

- 3.3 V?

- 5 V?

Control

Does it support:

- PWM?

- direction?

- enable?

- current control?

Protection

Does it have:

- overcurrent?

- overtemperature?

- undervoltage?

- short-circuit protection?

Thermal

Can it dissipate the expected heat?

26. A Common Integration Mistake

Suppose:

Motor:

24 V, 15 A stall

Driver:

24 V, 10 A maximum

Someone says:

"The motor is 24 V and the driver is 24 V, so they're compatible."

Wrong.

Voltage compatibility is only one part.

The current capability is also critical.

27. Another Common Mistake

Suppose:

MCU:

3.3 V logic

Driver:

5 V logic

Someone connects them directly.

Maybe it works.

Maybe it doesn't.

We must inspect the driver's datasheet:

What voltage counts as HIGH?

If:

then the MCU's 3.3 V HIGH may be recognized.

If:

then we may need:

level shifting

Never assume.

28. Encoder Selection

Now let's choose feedback.

We have several possibilities.

Incremental encoder

Outputs changes in position.

Typically:

A

B

Good for:

- speed

- relative position

- direction

But it generally doesn't inherently know absolute position after power loss.

29. Absolute Encoder

An absolute encoder reports an actual position.

For example:

0°

45°

90°

180°

270°

If power is removed and restored, it can still report the shaft's absolute position depending on encoder technology and system design.

Useful for:

- robotic joints

- precision positioning

- safety-critical positioning

30. Optical vs Magnetic

Optical encoder

Uses:

light + coded disk

Advantages:

- high resolution

- excellent precision

Potential disadvantages:

- contamination sensitivity

- mechanical complexity

Magnetic encoder

Uses:

magnetic field sensing

Advantages:

- compact

- robust

- good for many industrial environments

Potential limitations:

- magnetic interference

- resolution/performance varies by design

Again:

Environment matters.

31. Encoder Resolution

Suppose an encoder provides:

1000 pulses per revolution.

The interpretation depends on how the encoder and controller count edges.

But the basic idea is:

More resolution → finer measurement.

For a simplified 1000-count-per-revolution system:

per count.

But don't blindly equate encoder counts with final system accuracy.

Why?

Because accuracy also depends on:

- mechanical backlash

- sensor error

- mounting

- gearbox behavior

- noise

- control algorithm

32. This Is Another Systems Lesson

Suppose you buy a:

0.01° encoder.

Does that automatically give you:

0.01° robot joint accuracy?

No.

The mechanical system might have:

1° of backlash.

The encoder can't magically eliminate mechanical uncertainty unless the feedback is arranged appropriately and the control architecture addresses it.

So:

Sensor resolution ≠ system accuracy.

That's a very important engineering distinction.

33. Choosing a Microcontroller

Now let's move to the controller.

You've worked with:

Arduino Uno / ATmega328P.

That's an excellent learning platform.

But professional systems may use:

- STM32

- NXP

- TI

- Microchip

- ESP32

- RP2040

- other industrial MCUs

The question isn't:

"Which MCU is best?"

It's:

"What does the application require?"

34. MCU Selection Criteria

Consider:

CPU

- architecture

- clock speed

- performance

Memory

- Flash

- RAM

GPIO

How many pins?

ADC

How many channels?

What resolution?

Timers

How many?

Do they support:

- PWM?

- input capture?

- encoder mode?

Communication

Does it have:

- UART?

- SPI?

- I²C?

- CAN?

- USB?

- Ethernet?

Real-time requirements

Can it reliably execute the required control loops?

Power

How much does it consume?

Development ecosystem

Are:

- SDKs

- libraries

- debugging tools

- documentation

available?

35. Why STM32 Is Popular in Robotics

STM32 families offer many features useful for embedded control.

Depending on the specific model, you may have:

- multiple timers

- PWM

- ADC

- DMA

- UART

- SPI

- I²C

- CAN

- USB

- high CPU performance

This makes them suitable for:

motor control + sensors + communications

in a single embedded system.

36. DMA — An Important Concept

Here's a concept worth knowing.

DMA = Direct Memory Access

Instead of making the CPU manually move every piece of data:

Peripheral

↓

CPU

↓

Memory

DMA can allow:

Peripheral

↓

DMA

↓

Memory

while the CPU does other work.

This is useful for high-throughput systems such as:

- ADC streams

- UART reception

- SPI

- sensor data

It reduces CPU overhead.

37. Why This Matters for Robotics

Imagine you're reading:

- IMU

- encoder

- CAN

- ADC

while simultaneously running:

1 kHz control loop.

If the CPU spends all its time moving data around, you waste processing capacity.

Hardware peripherals + DMA can help keep the architecture efficient and deterministic.

This is why professional embedded systems are much more than:

digitalRead() and digitalWrite().

38. Choosing the Communication Interface

Now we have a system.

We need to decide how components communicate.

Let's compare our major options.

39. GPIO

GPIO is best when the information is simple.

Examples:

Button → MCU

Limit switch → MCU

Enable → Driver

Fault → MCU

It's basically:

HIGH / LOW

or sometimes input/output digital signaling.

Don't use a complicated communication protocol when all you need is:

"Is the switch pressed?"

40. UART

UART is useful for point-to-point serial communication.

MCU A

TX ───────── RX

RX ───────── TX

GND ──────── GND

MCU B

Common uses:

- debugging

- GPS

- Bluetooth modules

- embedded devices

It's simple and widely supported.

But basic UART isn't inherently a multi-drop bus like CAN.

41. I²C

I²C is useful for connecting multiple relatively nearby peripherals.

SDA ─────────────────────

SCL ─────────────────────

│       │        │

Sensor  Sensor   Sensor

Advantages:

- only two signal lines

- multiple devices

- addressing

Common uses:

- IMUs

- temperature sensors

- displays

- EEPROMs

But it's generally more suited to local board-level communication than a noisy long-distance robot network.

42. SPI

SPI is often used when you need:

high-speed local peripheral communication.

Conceptually:

MCU

│

├── SCLK

├── MOSI

├── MISO

└── CS

│

↓

Peripheral

Common uses:

- displays

- high-speed ADCs

- flash memory

- sensors

It's usually excellent for short distances.

43. CAN

Now we reach a protocol particularly relevant to robotics.

CAN is designed for robust communication in distributed systems.

Imagine:

CAN BUS

════════════════════════════════

│         │         │

Motor      Motor      Sensor

Ctrl       Ctrl       Node

This is very useful when you have multiple intelligent devices.

Examples:

- motor controllers

- BMS

- sensors

- distributed actuators

44. Why CAN Is So Useful in Robots

Imagine a robot with:

- 4 motor controllers

- BMS

- steering controller

- IMU

Instead of running separate point-to-point communication everywhere, a CAN bus can allow devices to share the network.

This reduces wiring complexity and supports distributed control architectures.

CAN also has strong error-detection mechanisms.

45. USB

USB is useful for:

- connecting a microcontroller to a computer

- cameras

- development/programming

- peripherals

For example:

ROS 2 Computer

│

USB

│

MCU

This can be convenient during development and in some deployed systems.

46. Ethernet

When you need:

- high bandwidth

- longer cable runs

- network connectivity

- multiple computers/devices

Ethernet becomes attractive.

A modern robot might have:

ROS 2 Computer

│

Ethernet

│

Network switch

┌──┼──┐

↓  ↓  ↓

Camera MCU LiDAR

This becomes particularly useful in larger systems.

47. How Do You Choose?

Don't memorize a table.

Ask:

What are the system requirements?

For example:

Simple switch

→ GPIO

IMU on same PCB

→ I²C or SPI

GPS module

→ UART

Multiple motor controllers distributed throughout robot

→ CAN

High-bandwidth camera

→ USB / Ethernet / specialized interface

Networked robot computers

→ Ethernet/Wi-Fi depending on requirements

48. The Decision Tree

When selecting an interface:

What data?

↓

How much data?

↓

How far?

↓

How many devices?

↓

How noisy is the environment?

↓

How reliable must communication be?

↓

What latency is acceptable?

↓

What hardware already exists?

↓

Choose interface

That is much better than memorizing:

"SPI is faster than I²C."

49. The Complete Component-Selection Picture

Now put everything together:

REQUIREMENT

│

┌───────────┼───────────┐

↓           ↓           ↓

Mechanical  Electrical   Software

│           │           │

↓           ↓           ↓

Motor       Battery      MCU

Gearbox     Driver       ROS 2

Encoder     Regulator    Protocol

│           │           │

└───────────┼───────────┘

↓

INTERFACES

↓

INTEGRATION

↓

TEST

↓

VALIDATION

And every component must fit the requirements of the whole system, not merely work by itself.

🧠 THE MOST IMPORTANT IDEA OF THIS CHAPTER

When you're given a component-selection question in Micro1, don't immediately name a component.

Show your reasoning.

For example:

Bad:

"I'd use an STM32."

Better:

"I'd first determine the required control-loop frequency, number of PWM channels, encoder interfaces, ADC channels, communication buses, memory requirements, and safety peripherals. If the system requires multiple timers for encoder capture and PWM, CAN communication, ADC inputs, and deterministic motor control, I'd select an MCU that provides those peripherals with sufficient processing and memory margin. An STM32 would be one candidate, but I'd confirm against the exact requirements and datasheet."

That sounds like an engineer.

🔥 NEXT CHAPTER — COMMUNICATION ARCHITECTURE

Now we're going to go deeper into something very important for the Micro1 Hardware Integrations role:

How do you design communication between an entire robot's components?

We'll build an actual network:

ROS 2 COMPUTER

│

Ethernet

│

┌─────┴─────┐

│   CAN BUS  │

└─────┬─────┘

│     │     │

MCU   BMS   Motor

│          Controller

Sensors          │

Motor

And we'll learn:

- CAN frames

- CAN IDs

- arbitration

- bus termination

- differential signaling

- baud rate

- CAN vs CAN FD

- message design

- heartbeat messages

- timeouts

- communication faults

- CRC/error detection

- bus-off

- how a ROS 2 computer talks to CAN devices

- how to debug a CAN network